Matlab2017a - Adams2012 Co-Simulation Documentation

Notice:

1. Must to make sure ADMAS and Matlab files are in the same folder with same directory;

2. Must be sure ADAMS contains file ‘adams_plant.mexw64’ in the following path:

C:\MSC.Software\Adams_x64\2012\controls\win64

3. Must be sure all the variables are named with ‘Full Name’, such as, ‘.RCB_CoSim.Variable’, otherwise, the ADAMS will display error ‘No model has been defined’;

Part I. Procedure for ADAMS

Step 1: Set the model in ADAMS

a. Prepare ‘Model.x_t’ file and import into ADAMS2012;

b. Add properties, such as, Mass, Joints, and Motions, or Markers, etc.

Step 2: Set ‘Inputs’ and ‘Outputs’ variables

a. Build – System Elements – State Variable – New;

b. Create all ‘Inputs’ variables, such as, inputs: [q0, q11, q12, q14, q21, q22, q23];

Don’t forget to add “Motion” on the actuated joints and the ‘Function (time)’ as:

VARVAL(.modelname.variablename)

c. Create all ‘Outputs’ variables, which could be a ‘measure’ we build in previous step;

- Open the variable ‘State Elements - Variable’,

- Select ‘Getting Object Data – Measure – Runtime Measure – Guess - Variable’,

- Press ‘Insert Object Name’ (Then, choose ‘Full Name’ is Very Important!!)

- Press ‘Ok’

Step 3: Build ‘ADAMS/Control’

a. Select ‘Plugin Manager – ADAMS/Control – Yes - OK’;

b. Name ‘Control_Plant_1’ as you wish;

c. Select Input Variables in ‘Input Signal(s)’ and ‘Output Signal(s)’;

d. Choose ‘Target Software – Matlab - OK’;

e. System will generate corresponding files in the default directory;

Part II. Procedure in Matlab

Step1: Calculate your variable value in Matlab ‘.m’ file;

a. Calculate the input variable value first by using your own file, it could be a time based vector or Matrix, which is used for inputs;

(Of course, you can also build Simulink block, such as, Sine/Cosine, etc., as inputs.)

Step2: Run ‘Controls_Plant_1.m’ file that ADAMS generated to build connection

a. Run ’Controls_Plant_1.m’ file;

b. Type ‘adams_sys’ in Command Window, you will get the following Simulink window;

Step3: Build your own time-based Simulink model

1) Type ‘adams_sys’

2) Copy ‘adams_sub ’ to your model

3) Construct your Simulink model

Step4: Set ‘MSC.software’ in ‘adams_sub’

a. Only need to set ‘Animation Mode - Iterative’;

Step5: Go back to home page of your Simulink model, and set an appropriate simulation time;

a. Run simulation with appropriate time

b. the Co-Simulation will launch the ADAMS to proceed the calculation

Co-Simulation is launched:

NOTE:

As my case for example, the input is a 1X100 vector with each step assigned with time; I use ‘From Workspace’ block that can send the vector value of each step to the ‘adams_sub’, see the following example, if I have 10 variables with 10X100 matrix.

My Solution:

a. Set a time-based Martix in ‘From Workspace’:  ‘q0q1q2SlideLeftRightArm_time.mat’;

With the first column is Time, then followed with each variable column;

b. Distribute 10 variables by using ‘Demux’

Part III. Import simulation result in ADAMS

After running your co-simulation, you can load your calculation results into ADAMS which located in your default directory, and then, you can see the results;